Installer Manual

SPARK 310 - IP wall-mounted outdoor station

Surface-mounted SPARK 310 outdoor station with 3 buttons and display. Equipped with HD camera with panoramic view and QR code reading, motion sensor radar, Ethernet and Wifi network interfaces, Bluetooth and NFC access control interfaces, programmable physical key. Features 1 relay and one powered output, as well as 2 inputs and 485 bus interface IP65 rating Available accessories: facial recognition cameras, LTE card for cellular connection, RF module for remote control function, DDA interface for deaf, memory extension via SD card for local recording.

PRODUCT ID spark-310 CATEGORY door-stations LANGUAGE EN VERSION 1.0

INTENDED USE · SAFETY · COMPLIANCE

Safety and compliance

This page summarises the warning blocks commonly included in professional installer manuals for video door entry entrance panels. Symbols, directives and wording must always be aligned with the technical file of the specific model.

Common symbols / replace with official artwork in the final release

These badges are layout placeholders for the manual. CE, WEEE and other regulatory marks should be replaced with the manufacturer’s official vector artwork in the final version.

| READ Read the manual before installation | ATTN Important warnings and cautions |
| --- | --- |
| TECH Qualified technical personnel only | OFF Disconnect power before wiring |
| CE CE marking where applicable | WEEE Separate collection / WEEE |

SAFETY AND COMPLIANCE

| Intended use Device intended for audio/video communication and access control systems in residential, commercial, industrial and public buildings. Any different use is considered improper. | Qualified installation Installation, configuration, commissioning and non-routine servicing must be carried out by qualified personnel in accordance with the electrical regulations in force in the country of installation. |
| --- | --- |
| Power and wiring Disconnect power before working on the wiring. Use suitable cables and respect polarity, cross-section and distance limits. Avoid unplanned mixing with mains power lines. | Cleaning and maintenance Clean with a soft dry cloth or a slightly damp cloth. Do not use alcohol, solvents, sprays or aggressive chemicals. Do not open or tamper with the device unless expressly permitted by the manufacturer. |
| Compliance documentation CE marking, EU declarations, applicable directives and radio data must match the technical file of the specific model. Documentation should state where the full declaration of conformity is available. | End-of-life disposal At the end of its life, the product must be delivered to separate collection channels for electrical and electronic equipment. WEEE information and disposal instructions should be included where applicable. |

For versions with Wi‑Fi, Bluetooth, LTE or other radio interfaces, add the EU declaration of conformity reference and the essential radio information required by the model technical file.

Recommended contents inspired by recurring safety sections found in installer manuals from leading video door entry manufacturers.

Surface-mounted SPARK 310 outdoor station with 3 buttons and display. Equipped with HD camera with panoramic view and QR code reading, motion sensor radar, Ethernet and Wifi network interfaces, Bluetooth and NFC access control interfaces, programmable physical key. Features 1 relay and one powered output, as well as 2 inputs and 485 bus interface IP65 rating Available accessories: facial recognition cameras, LTE card for cellular connection, RF module for remote control function, DDA interface for deaf, memory extension via SD card for local recording.

# Table of contents

# 1. Overview

## 1.1 Description

Surface-mounted SPARK 310 outdoor station with 3 buttons and display. Equipped with HD camera with panoramic view and QR code reading, motion sensor radar, Ethernet and Wifi network interfaces, Bluetooth and NFC access control interfaces, programmable physical key. Features 1 relay and one powered output, as well as 2 inputs and 485 bus interface IP65 rating Available accessories: facial recognition cameras, LTE card for cellular connection, RF module for remote control function, DDA interface for deaf, memory extension via SD card for local recording.

## 1.3 Key features

- Interface with 3 direct call buttons

- Multiple interfaces for access control (QR,NFC,Bluetooth,Facial Recognition

- High-quality audio communication using DSP for audio filters

- Video communication with panoramic view camera that also frames sideways

- Extremely solid metal construction and design

# 2. Technical specifications

Technical specifications are provided in tables (source: product specs).

## General

| Parameter | Value |
| --- | --- |
| Model | SPARK 310 |
| Type | External IP video intercom device |
| Installation | Exterior Wall |
| Available colors | Gray Shadow,Silver |
| Height | 187 mm |
| Width | 113 mm |
| Thickness | 30 mm |
| Weight | 850 g |
| Material | Painted metal |
| Working environments | -20°C / +55°C |

## Power supply

| Parameter | Value |
| --- | --- |
| PoE | 802.3af |
| External power supply | 12/24 V DC @ 1 A |
| Consumption | 2.5 W (Standby) / 5 W (Max) |

## Display

| Parameter | Value |
| --- | --- |
| Dimensions | 2.4" |
| Touch screen | NO |
| Resolution | 240 × 320 |
| Brightness | 800 cd/m² |
| Type | Bar |

## Video camera

| Parameter | Value |
| --- | --- |
| Resolution | 2576 × 1456 |
| Viewing angle | H=170° - V=90° |
| Sensor | 1/3" CMOS |
| Sensitivity | Color 0.02 Lux - B/W 0 Lux with active IR |
| IR illumination | 4 LED IR coverage 5 m |

## Audio

| Parameter | Value |
| --- | --- |
| Speaker | 1 × 2 W |
| Microphones | 2 Digital |
| DSP | AEC, ANS, AGC |

## Network and connectivity

| Parameter | Value |
| --- | --- |
| Ethernet | RJ45 10/100 Mbps |
| Wi-Fi | Dual band 2.4/5 GHz IEEE 802.11 b/g/n/ac |
| Bluetooth | 4.2 / BLE |
| NFC | 13.56 MHz, MIFARE Classic, Ultralight, Plus, DESFire |
| RS485 | Master/Slave, ASCII/Modbus RTU |
| LTE | Optional |

## Inputs and outputs

| Parameter | Value |
| --- | --- |
| Lock output | 12 V DC @ 1 A impulsive |
| Auxiliary relay | NO dry contact, 24 V DC @ 2 A max, monostable 1-30 sec. |
| Inputs | 2 opto-isolated Pull-up inputs |

## Integrated sensors

| Parameter | Value |
| --- | --- |
| Movement | Radar, distance and direction up to 5 m |
| Vandalism | Shock and vibration sensor |
| Tampering | Forced opening sensor |

## Certifications

| Parameter | Value |
| --- | --- |
| Protection | IP65 / IK08 |

## Optional Modules

| Parameter | Value |
| --- | --- |
| LTE Connectivity | LTE module for data connectivity |
| Facial recognition | Dual camera module for access control based on face recognition |
| Hearing aid interface | HL/IDL audio induction circuit |
| RF remote control | 433 MHz transceiver module |

# 3. Installation

## 3.1 Introduction

- Front LED light

- Proximity radar

- Panoramic camera

- Touch screen

- Metal keys

- Brightness sensor

- Microphones

- Infrared led

- Facial recognition cameras

- NFC reader

- Lights of the lower

- RGB color illuminated logo

## 3.2 Mechanical installation

### Wall support

Step 1 — Preparation

Place the DIMA on the wall,with the holes at the top at a height from the ground between 155 and 165 cm, making sure that the "TOP" side is facing up. Draw with a pencil the positions of the four holes highlighted in red in the figure.

Step 2 — Check

Before proceeding, carefully check that there are no cables or pipes behind the wall, then drill holes in the wall at the previously drawn points.

Step 3 — Drilling

Insert the dowels making sure they are perfectly flush with the wall. Then apply the fixing plate and secure it using the four screws provided, as shown in the figure.

Step 4 — Installation support

To facilitate connections to the terminal block, a stand is available to support the plate during wiring operations. Attach the stand to the plate and tighten the front screw, as shown.

Step 5 — Plate fixing

Then proceed by hooking the stand to the plate and connect the cables to the terminal block. Once the connections are completed, unhook the stand from the plate and then from the device.

Step 6 — Fixing device

Hook the plate onto the plate and screw in the two front screws as shown in the figure.

Step 7 — Closing

To complete the installation, insert the cover from bottom to top until you hear it click.

Step 8 — Disassembly

To uninstall the plate, insert the safety wrench into the slot on the bottom of the cover, push the cover down, and unscrew the front screws.

## 3.3 Wiring

Step 1 — Preparation

WIRING ACCESS: After unscrewing the cover, remove the front screw at the bottom and rotate the holder so that the terminal area can be accessed. Locate the PoE input and other power terminals (as needed as described in the next steps).

Step 2 — Optional Power Supply

OPTIONAL POWER SUPPLY: If the Ethernet network is not PoE, a 24VDC/1A power supply must be used. Connect the power supply to the terminals indicated.

Step 3 — Direct connection Lock

LOCK OUTPUT: An electric lock can be directly connected to the LOCK+ / LOCK- terminals (max 1A, cable ≤30m, 1.5mm²). An intermediate relay must be used for different loads, as shown in the application diagrams.

Step 4 — Powered Lock Connection

AUX OUTPUT: In this case the output is a dry contact switch that can drive an external load. If the load is in 220VAC, a power relay must be used.

Step 5 — Connecting Button Inputs

INPUTS: Up to two inputs (e.g. external door opener buttons) can be connected. The inputs are programmable. Example of external door opener button on INPUT 1 / GND.

# 4. Operating notes

## 4.1 Maintenance

- Clean with a soft cloth.

- Do not use solvents.

# 5. Revisions

| Version | Notes |
| --- | --- |
| 1.0 | 2026-04-06 |